[ Your company name and logo ] Replace this box with your company details before issuing.

# Steel Erection RAMS

A free, editable RAMS focused on steel erection — lifting, temporary stability, bolt-up and work at height. Add your company details above, edit the example hazards and steps, and delete the grey guidance before issuing.

## Project details

| Company name |  |
| --- | --- |
| Project / site |  |
| Client / principal contractor |  |
| Document reference |  |
| Prepared by |  |
| Date prepared |  |
| Revision |  |
| Review / expiry date |  |

## 1. Scope of works

What is being erected, where, the limits of your work and what is excluded.

## 2. Risk assessment

Score Likelihood (1–5) × Severity (1–5) using the matrix. 1–6 low, 8–12 medium, 15–25 high. Example erection hazards are pre-filled — edit to suit and delete this guidance.

| L ↓ / S → | Severity 1 | Severity 2 | Severity 3 | Severity 4 | Severity 5 |
| --- | --- | --- | --- | --- | --- |
| Likelihood 1 | 1 | 2 | 3 | 4 | 5 |
| Likelihood 2 | 2 | 4 | 6 | 8 | 10 |
| Likelihood 3 | 3 | 6 | 9 | 12 | 15 |
| Likelihood 4 | 4 | 8 | 12 | 16 | 20 |
| Likelihood 5 | 5 | 10 | 15 | 20 | 25 |

| Hazard | Who is at risk | Existing control measures | Risk L×S=R | Further controls needed |
| --- | --- | --- | --- | --- |
| Working at height — falls (example) | Erectors | Edge protection / nets where practicable; fall arrest harness clipped to a suitable point; MEWP where possible | 4×5=20 | Rescue plan; harness/lanyard inspection; competent operatives |
| Lifting operations — struck by load / crane overturn (example) | Everyone in zone | Appointed person; lift plan; certified accessories; exclusion zone; trained slinger/signaller | 3×5=15 | Check ground bearing; wind limits; LOLER & BS 7121 |
| Temporary instability / collapse before bracing (example) | Erectors, others | Erect in designed sequence; temporary bracing/guys as per design; no load until secured | 3×5=15 | Engineer-approved temporary works; hold points |
| Dropped objects / tools at height (example) | People below | Tool tethers; exclusion zone below; no loose materials at height | 3×4=12 | Toe boards / nets; brief ground crew |

## 3. Sequence of work

Set out the erection sequence step by step. Example shown — edit to suit.

- Deliveries, offload and set down with agreed lifting methods.

- Set up exclusion zone, access and edge protection.

- Lift and land columns/beams in the designed sequence.

- Fit temporary bracing / guys to keep the frame stable.

- Bolt up / weld connections; torque check or weld inspection.

- Remove temporary works once permanently stable; final inspection and handover.

## 4. Lifting operations

Record the lift plan details. Comply with LOLER and BS 7121.

| Appointed person |  |
| --- | --- |
| Crane / lifting equipment |  |
| Heaviest load & radius |  |
| Lifting accessories (cert. dates) |  |
| Ground / outrigger arrangements |  |
| Wind limit for lifting |  |

## 5. Plant, equipment & access

List each item and its inspection status.

| Item / equipment | Inspection or PAT date | Operator & competence |
| --- | --- | --- |

## 6. PPE required

Mark what is required, including fall-arrest harness for work at height.

| PPE item | Required (Y/N) | Notes |
| --- | --- | --- |
| Hard hat |  |  |
| Safety boots |  |  |
| Hi-vis |  |  |
| Gloves |  |  |
| Eye protection |  |  |
| Hearing protection |  |  |
| RPE (FFP3 / powered APF 20) |  |  |
| Welding PPE (leathers / gauntlets) |  |  |

## 7. Permits & isolations

| Permit | Required (Y/N) | Reference / number |
| --- | --- | --- |
| Hot works permit |  |  |
| Permit to work |  |  |
| Confined space |  |  |
| Lifting permit |  |  |

## 8. Emergency arrangements

Include a rescue-from-height plan for anyone working in a harness or MEWP.

| Role | Name | Contact |
| --- | --- | --- |
| First aider |  |  |
| Site supervisor |  |  |
| Nearest A&E |  |  |
| Emergency services |  |  |
| Out-of-hours contact |  |  |

## 9. Briefing & sign-off

Everyone carrying out this work must read and understand this RAMS before starting.

| Name (print) | Signature | Date |
| --- | --- | --- |

Tired of re-typing this for every job? Fabora RAMS turns templates like this into reusable, editable records you build in minutes — with saved hazard, COSHH, PPE and equipment libraries, branded PDF export, share links and revision control, instead of editing the same Word file every time. Start free (no card) at faboraplatform.com · This is a practical drafting aid. Final review, suitability and approval remain with your business.