[ Your company name and logo ] Replace this box with your company details before issuing.

# Welding & Hot Works Risk Assessment

A free, ready-to-edit risk assessment for welding and hot works in UK fabrication and site work. Add your company details above. The hazard rows below are pre-filled with common welding risks and controls — review every one, edit to match your job, and delete the grey guidance before issuing.

## Job details

| Company name |  |
| --- | --- |
| Project / site |  |
| Client / principal contractor |  |
| Document reference |  |
| Prepared by |  |
| Date prepared |  |
| Revision |  |
| Review / expiry date |  |

## Risk assessment

Score Likelihood (1 = rare … 5 = almost certain) and Severity (1 = minor … 5 = fatal). Risk = Likelihood × Severity. 1–6 low (monitor), 8–12 medium (add controls), 15–25 high (do not proceed until reduced). Use the matrix below, then complete the table. Replace the example rows with your own and delete this guidance before issuing.

| L ↓ / S → | Severity 1 | Severity 2 | Severity 3 | Severity 4 | Severity 5 |
| --- | --- | --- | --- | --- | --- |
| Likelihood 1 | 1 | 2 | 3 | 4 | 5 |
| Likelihood 2 | 2 | 4 | 6 | 8 | 10 |
| Likelihood 3 | 3 | 6 | 9 | 12 | 15 |
| Likelihood 4 | 4 | 8 | 12 | 16 | 20 |
| Likelihood 5 | 5 | 10 | 15 | 20 | 25 |

| Hazard | Who is at risk | Existing control measures | Risk L×S=R | Further controls needed |
| --- | --- | --- | --- | --- |
| Welding fume (incl. mild steel fume — a carcinogen, HSE STSU1-2019) | Welders, others nearby | Local exhaust ventilation (LEV) at the arc; RPE (FFP3 / powered RPE APF 20); general ventilation; LEV thoroughly examined | 3×4=12 | LEV examination in date (≤14 months); fit-tested RPE; COSHH Reg 7 review |
| Fire / hot work | Everyone on site | Hot works permit; remove or cover combustibles 10 m+; suitable extinguisher to hand; fire watch during and 60 min after | 3×4=12 | Confirm no work near flammables; check permit close-out |
| Arc eye / UV radiation | Welders, passers-by | Correct-shade auto-darkening filter; welding screens/curtains around the bay | 3×3=9 | Brief others to keep clear; check screens in place |
| Electric shock | Welders | PAT-tested set and leads; dry conditions; good return lead connection; no damaged cables | 2×4=8 | Inspect leads before use; remove damaged equipment |
| Burns — hot metal & spatter | Welders | Leather gauntlets, welding apron/leathers, boots; allow metal to cool; spatter guards | 3×2=6 | Brief on cool-down; no synthetic clothing |
| Hand-arm vibration (grinding/prep) | Operatives | Maintained, low-vibration tools; limit trigger time; rotate tasks; report symptoms | 3×3=9 | Use the HAVS calculator to track daily exposure |
| Gas cylinders (if oxy-fuel / shielding gas) | Operatives | Cylinders secured upright; flashback arrestors; correct storage; leak checks | 2×4=8 | Check storage/segregation; close valves after use |

## Welding fume — key control note

Since HSE bulletin STSU1-2019, all indoor welding of mild steel requires effective engineering controls (LEV); where LEV is not adequate, suitable RPE is also required, and RPE is required for outdoor welding. Control is a legal duty under COSHH Regulation 7.

## PPE required

Mark what is required for this job.

| PPE item | Required (Y/N) | Notes |
| --- | --- | --- |
| Hard hat |  |  |
| Safety boots |  |  |
| Hi-vis |  |  |
| Gloves |  |  |
| Eye protection |  |  |
| Hearing protection |  |  |
| RPE (FFP3 / powered APF 20) |  |  |
| Welding PPE (leathers / gauntlets) |  |  |

## Briefing & sign-off

Everyone carrying out welding/hot works must read and understand this assessment before starting.

| Name (print) | Signature | Date |
| --- | --- | --- |

Tired of re-typing this for every job? Fabora RAMS turns this template into reusable, editable RAMS you build in minutes — with saved hazard, COSHH, PPE and equipment libraries, branded PDF export, share links and revision control, instead of editing the same Word file every time. Start free (no card) at faboraplatform.com · This is a practical drafting aid. Final review, suitability and approval remain with your business.